HUSKEY POCKET / KEYVAULT

Operation Manual, Architecture Specification, and Technical Reference

Platform: Waveshare ESP32-S3-Touch-LCD-2.8 | Reference firmware: 1.0.1

SCOPE AND STATUS / Huskey Pocket stores the vault locally and does not use cloud services. This project has not received a security audit. Vault encryption and Secure Boot are part of the device configuration, but they do not certify its security.

1. Overview and Hardware Architecture

The device combines control, storage, and display hardware to run the credential manager locally.

1.1 Microcontroller Specifications

Espressif ESP32-S3 CPU: dual-core 32-bit Xtensa LX7 processor clocked up to 240 MHz.

Flash memory: 16 MB according to this repository’s build configuration.

PSRAM: 8 MB of pseudo-static RAM for LVGL graphics buffers, font decoding, and cryptographic processing.

Cryptographic acceleration: vector extensions accelerate SHA-256 hashing and AES encryption routines.

1.2 Display and Touch System

ST7789 display: 2.8-inch IPS TFT LCD with 240 × 320 resolution and 16-bit color (RGB565). The controller communicates over a 40 MHz SPI bus with direct line rendering.

CST328 touch panel: capacitive digitizer with an independent I2C interface (secondary Wire1 bus). Supports multitouch, continuous swipe gestures, and navigation shortcuts.

1.3 Integrated Sensors and Peripherals

Real-time clock (PCF85063): requires an external backup battery to retain time while the main 523450 LiPo battery is off.

QMI8658C inertial sensor: six-axis IMU (three-axis accelerometer and gyroscope) over I2C. Monitors dynamic forces for Snatch Lock.

MicroSD storage: slot for cards using 1-bit SD_MMC mode.

Power management: AO3401 P-channel MOSFET cuts power to the display and secondary components during deep sleep.

Haptic feedback: ERM motor with transistor driver provides physical confirmation pulses for key presses and alerts.

2. Cryptographic Architecture and Security

Sensitive vault fields are encrypted with AES-256-CBC. CBC does not include built-in cryptographic authentication; verification is performed separately.

2.1 Master Key Derivation

The unlock PIN contains six numeric digits.

The PIN is not stored as a persistent plaintext preference; the firmware handles it temporarily in memory during authentication.

The vault key is derived from the PIN using PBKDF2-HMAC-SHA256 with 50,000 iterations and a 16-byte salt. The .hky container uses 100,000 iterations for its export password.

Derived key (DK): the 256-bit result is used as the symmetric encryption key to open and decrypt the internal database.

2.2 Vault Encryption

Symmetric encryption: credential records are encrypted individually with AES-256-CBC, PKCS#7 padding, and a cryptographically random initialization vector (IV) for each entry.

PIN verification uses an encrypted control value. The firmware is not claimed to be free of timing leakage.

After five consecutive incorrect vault PIN attempts, the firmware starts a factory reset. FRP retains the previous PIN and applies its own attempt limit.

2.3 Partitioning and Data Erasure

Data partition: entries reside in the internal Flash LittleFS partition. The vault is stored in a LittleFS partition on internal Flash.

Factory reset removes the vault and local preferences while retaining FRP. Logical cleanup does not guarantee physical overwriting of every Flash cell.

3. Active Protection Systems

In addition to encryption at rest, the device has reactive safeguards based on physical sensors and custody timers.

3.1 Snatch Lock

This system is intended to prevent unauthorized access when the device is unlocked in the user’s hand.

Continuous monitoring: the microcontroller periodically reads linear acceleration and angular rotation registers from the QMI8658C.

Motion-pattern detection: when the combined acceleration and gyroscope magnitudes exceed their thresholds, the system interprets the movement as a possible forceful grab or drop.

Snatch Lock polls the QMI8658C IMU in software and locks the device when readings exceed the selected thresholds. No response time is guaranteed.

Adjustable levels: choose Off, Low, Medium, or High sensitivity in Settings.

3.2 Dead Man’s Switch (Lost Mode)

Designed for situations involving loss, abandonment, or prolonged inability to access the device.

Lost Mode is configured in days. If the period expires without a genuine unlock, the firmware starts a factory reset. The RTC must retain a valid time.

Each successful authenticated unlock resets the Dead Man’s timer recorded by the RTC.

When the configured period expires, the firmware starts a factory reset. The feature depends on the RTC retaining a valid time.

3.3 Auto-lock on Inactivity

Choose 30 seconds, 1 minute, 2 minutes, 5 minutes, or Never. Locking the session clears the session key from memory.

4. HID Keyboard Emulation and Connectivity

Huskey Pocket can act as a standard text-input device for computers and smartphones.

4.1 USB HID Mode

When connected over USB-C, the ESP32-S3 enumerates as a physical keyboard through the native USB stack. No client software, extensions, or host drivers are required.

Keystrokes are sent using HID keyboard reports.

4.2 Bluetooth Low Energy (BLE) Mode

Wireless HID keyboard emulation over GATT using NimBLE-Arduino.

Supports encrypted pairing and stores bonding keys for reconnection.

Compatible with Windows 10/11, macOS, Linux, Android, and iOS.

4.3 Keyboard Layouts

Select the target computer’s keyboard layout to type special characters as stored:

ES-LAT: Latin American Spanish, including accents, ñ, at sign, and symbols according to the Latin American keyboard map.

ES-ES: Spain Spanish according to the standard Spanish ISO layout.

US: United States English ANSI layout.

4.4 One-touch Typing

Username + Tab: types the username and sends Tab to move the cursor to the password field.

Password + Enter: types the password and sends Enter to submit the sign-in form.

TOTP typing: sends temporary verification codes directly to a sign-in form.

Generator typing: types a newly generated password on the host with one button press.

5. Account and Credential Management

The main interface organizes and manages the credential database using touch controls.

5.1 Record Types

Web accounts and services: structured fields for service name, username, password, optional notes/URL, and TOTP secret.

Secrets / Notes Only: intended for wallet recovery phrases, SSH keys, backup codes, or longer personal notes.

5.2 Folders and Categories

Hierarchical structure: classify accounts into folders such as Work, Finance, Personal, or Servers.

Create, rename, or delete categories. Deleting a folder moves its accounts to Uncategorised instead of deleting them.

5.3 Search and Favorites

Live search: incremental filtering as characters are typed.

Favorites: mark records with a star to show them at the top of the list or in a dedicated tab.

5.4 Protected Display and QR Sharing

Passwords are masked by default. An eye button temporarily reveals the text and provides haptic feedback.

QR transfer: displays a high-contrast black-and-white QR code containing entry data. A phone camera can scan it without electrical contact or wireless transmission.

6. Two-Factor Authentication (TOTP / 2FA)

Huskey Pocket stores TOTP secrets and generates six-digit codes every 30 seconds. It does not necessarily replace other authentication methods.

6.1 Operation and Time Accuracy

RFC 6238: HMAC-SHA1, a 30-second time step, and a six-digit output.

The service-provided secret is stored in Base32 format.

Calculations use the timestamp from the external PCF85063 RTC without requiring internet time synchronization (NTP). An external backup battery is required to retain time while the device is off.

6.2 Token Display

The screen shows the six-digit code with a space separator (for example, 492 810) for readability.

A graphical and numeric indicator counts down the remaining seconds before the code expires and a new one is generated.

A keyboard button sends the current numeric code to the computer over USB or Bluetooth.

7. Built-in Password Generator

Creates random strings for new records or periodic password changes.

7.1 Configuration Options

Adjustable length: 8 to 32 characters using an interactive slider.

Character sets: independent toggles for uppercase (A–Z), lowercase (a–z), numbers (0–9), and special symbols (!@#$%...).

Ambiguity filter: optionally omit visually confusable characters such as 1, lowercase l, uppercase I, 0, and uppercase O.

7.2 Use Modes

Standalone mode: generate a password from the main menu and type it into a host application.

Form-embedded mode: generator button beside the password field in account creation and editing forms.

8. On-screen Touch Keyboards

The system offers two keyboard layouts for text entry on the compact 2.8-inch screen.

8.1 KeyVault Split Keyboard

Six horizontal sliding panels: left and right letter panels, plus left and right panels for Symbols 1 and Symbols 2.

Smart Shift: one tap capitalizes the next character; a second tap enables Caps Lock.

Animated horizontal transitions between panels.

8.2 T9 Grid Keyboard (4×4)

A 16-button grid inspired by classic mobile phone keypads.

Multitap: rapid repeated presses cycle through the letters assigned to a key (for example, key 2 cycles through A, B, and C).

Long press: holding a key enters its number or shortcuts such as .com or a space.

Switch between the split keyboard and T9 at any time in Settings.

9. Backups, Export, and USB Disk Mode

Tools for backing up information, migrating data, or connecting the memory card to a computer.

9.1 Encrypted Backups (.hky — Huskey Backup)

Encrypted .hky backup: stores vault information in an encrypted container; the format does not imply compression.

Separate password: the backup uses AES-256 with a key derived from a password set during export. It may be the same as or different from the PIN.

Backups are stored in /export on the microSD card.

9.2 Plain CSV Export

Creates a comma-separated text file compatible with desktop managers such as KeePass, Bitwarden, and Excel.

A confirmation screen warns that CSV data will be readable as plain text on the SD card.

9.3 Asynchronous, Non-blocking Import

Restore encrypted .hky files or import external .csv files placed on the microSD card.

Sequential timer steps and a live progress bar keep the interface responsive when importing files with hundreds of entries.

9.4 Backup Cleanup and USB Mass Storage (MSC)

Cleaning /export overwrites and deletes managed files. Overwriting does not guarantee physical removal of every copy from Flash memory.

USB Mass Storage mode exposes the microSD card through USB-C as a removable drive without removing the physical card.

10. User Interface and Experience (LVGL 8.x)

The interface uses LVGL v8.3.6, adapted for rendering on microcontrollers with PSRAM.

10.1 Status Bar

The top bar on working screens displays battery charge in four segments (25%, 50%, 75%, 100%) and animates while charging.

Live clock: continuously displays the current time in 24-hour format using the RTC.

Status icons indicate microSD presence and USB/BLE connection status.

10.2 Theme and Color Engine

Dark theme: dark palette designed for low-light comfort.

Light theme: light backgrounds and black text for readability outdoors.

Custom theme: adjust hue and saturation for the background, cards, and accent buttons. The system calculates luminance and chooses white or black text accordingly.

10.3 Navigation and Feedback

Toast notifications: brief notices (Confirmed, Deleted, Error) appear near the bottom without interrupting the workflow.

Floating power panel: circular menu with shortcuts to shut down, restart, or lock the device.

Onboarding: first-boot setup flow to set the master PIN, select a language, and set the hardware clock.

11. Troubleshooting and Maintenance

Technical recommendations for reliable device operation.

11.1 Incorrect PIN Lockouts

After an incorrect PIN, the system imposes a wait before the next attempt. There is no PIN recovery backdoor by design; a factory-reset sequence is required to reset the device.

11.2 microSD Card Requirements

The microSD card must use a format compatible with SD_MMC and the filesystem configured by the firmware. The code documentation does not specify recommended card capacities or allocation unit sizes.

11.3 Battery Charging and Care

Main LiPo battery: the enclosure is designed for a 3.7 V, 523450-format battery. Consult the board specifications for information about its charging circuit.